# Egyptian License Plate Recognition System

Complete Project Documentation

# Table of Contents

1. Project Overview

2. Dataset Preparation

3. Model Training

4. Backend API

5. Dashboard

6. Hardware Setup

7. Deployment

8. System Architecture

# Egyptian License Plate Recognition System

## Project Overview

This project presents a complete end-to-end solution for automatic Egyptian license plate recognition, combining IoT hardware with deep learning computer vision. The system detects vehicles using an ultrasonic sensor, captures images with an ESP32-CAM microcontroller, processes them through a cloud-based AI backend, and displays results on a real-time monitoring dashboard.

[IMAGE PLACEHOLDER]
Project Overview Diagram

Caption: High-level system architecture showing the flow from vehicle detection to dashboard display

## Problem Statement

Egyptian license plates present unique challenges for automated recognition systems:

Arabic Script: Plates contain Arabic letters and Eastern Arabic numerals (٠١٢٣٤٥٦٧٨٩)

Standardized Format: Egyptian plates follow a specific 6-7 character format

Variable Conditions: Real-world capture involves varying lighting, angles, and distances

Limited Datasets: Few publicly available datasets exist for Egyptian plate recognition

## Solution Approach

Our solution combines three main technologies:

IoT Hardware Layer: ESP32-CAM with ultrasonic proximity detection for automatic vehicle sensing

AI/ML Backend: Custom-trained YOLOv10 models for plate detection and Arabic OCR

Web Dashboard: Real-time monitoring interface displaying all detected plates

## Technology Stack

| Layer | Technology | Purpose |
| --- | --- | --- |
| Hardware | ESP32-CAM | Image capture and WiFi transmission |
| Hardware | HC-SR04 Ultrasonic | Vehicle proximity detection |
| ML Framework | YOLOv10 | License plate detection & character recognition |
| Backend | FastAPI (Python) | REST API for image processing |
| Database | PostgreSQL (Neon) | Detection records storage |
| Storage | Cloudinary | Image CDN hosting |
| Frontend | React + TypeScript | Real-time dashboard |
| Deployment | Railway | Cloud hosting platform |
| Annotation | Roboflow | Dataset labeling and augmentation |

## Project Components

[IMAGE PLACEHOLDER]
Components Diagram

Caption: Visual breakdown of the four main project components

### 1. Computer Vision & ML

Custom dataset of 1,800 Egyptian license plate images

YOLOv10 model trained for plate detection (97.5% mAP@50)

Arabic OCR model for character recognition

### 2. Backend API

FastAPI server handling image processing requests

Three-stage detection pipeline (detect plate → crop → OCR)

Async database operations with PostgreSQL

### 3. Hardware System

ESP32-CAM microcontroller with OV2640 camera

Ultrasonic sensor for automatic capture triggering

Battery-powered portable operation

### 4. Monitoring Dashboard

React-based real-time interface

Live detection feed with 30-second auto-refresh

Dark theme optimized for monitoring

## Live Deployments

| Service | URL |
| --- | --- |
| API Backend | https://robotics-egyptian-lpr-production.up.railway.app |
| Dataset | https://universe.roboflow.com/vguard-vxduz/egyptian-license-plate-detection |

## Key Achievements

95.88% Precision in license plate detection

93.98% Recall rate for plate localization

97.50% mAP@50 detection accuracy

Real-time Processing with cloud-based inference

Arabic Character Support for authentic Egyptian plates

## Documentation Structure

This documentation is organized into the following sections:

Overview (this document)

Dataset Preparation - Data collection and annotation process

Model Training - YOLOv10 training methodology and results

Backend API - FastAPI server architecture

Dashboard - React frontend implementation

Hardware Setup - ESP32-CAM circuit and connections

Deployment - Cloud infrastructure setup

System Architecture - Complete data flow and integration

# Dataset Preparation

## Overview

The foundation of our Egyptian License Plate Recognition system is a custom dataset specifically created for Egyptian plates. This section documents the data collection, annotation, and augmentation processes.

## Dataset Statistics

| Metric | Value |
| --- | --- |
| Total Images | 1,800+ |
| Annotation Type | Bounding boxes |
| Platform | Roboflow |
| Classes | License plate region |
| Format | YOLOv10 compatible |

Dataset Link: Egyptian License Plate Detection on Roboflow Universe

## Data Collection Process

### Image Sources

Images were manually collected from various sources to ensure diversity:

Different vehicle types (cars, trucks, buses, motorcycles)

Various lighting conditions (daylight, evening, artificial lighting)

Multiple angles and distances

Different Egyptian governorates and plate styles

[IMAGE PLACEHOLDER]
Sample Dataset Images

Caption: Sample images from the collected dataset showing variety in angles, lighting, and plate types

### Egyptian License Plate Characteristics

Egyptian license plates follow a standardized format:

Character Count: 6-7 characters

Script: Arabic letters and Eastern Arabic numerals

Layout: Letters on left, numbers on right

Colors: White background with blue/black text (private vehicles)

[IMAGE PLACEHOLDER]
Egyptian Plate Format

Caption: Standard Egyptian license plate format showing character arrangement

## Annotation Process

### Roboflow Annotation Tool

All 1,800 images were manually annotated using Roboflow's annotation interface:

Bounding Box Annotation: Each license plate region was marked with a rectangular bounding box

Class Labeling: Single class "license_plate" for detection

Quality Control: Manual review of all annotations for accuracy

[IMAGE PLACEHOLDER]
Annotation Example

Caption: Roboflow annotation interface showing bounding box placement on a license plate

### Annotation Guidelines

Boxes tightly fit around the plate region

Include slight padding for edge characters

Mark partially visible plates when >50% visible

Exclude severely occluded or blurry plates

## Dataset Augmentation

Three different augmentation strategies were tested to determine optimal training conditions:

### Version 1: Noise-Blur-Crop

| Augmentation | Value |
| --- | --- |
| Crop | 0-20% zoom |
| Blur | Up to 1px |
| Noise | Up to 1.5% pixels |

### Version 2: Rotation-Shear-Noise

| Augmentation | Value |
| --- | --- |
| Rotation | -10° to +10° |
| Shear | ±10° horizontal & vertical |
| Noise | Up to 1.5% pixels |

### Version 3: Full Augmentation

| Augmentation | Value |
| --- | --- |
| Rotation | -10° to +10° |
| Shear | ±10° horizontal & vertical |
| Brightness | -20% to +20% |
| Exposure | -10% to +10% |
| Blur | Up to 1px |
| Noise | Up to 1.5% pixels |

[IMAGE PLACEHOLDER]
Augmentation Comparison

Caption: Visual comparison of the three augmentation strategies applied to the same source image

## Dataset Split

The dataset was divided into training, validation, and test sets:

| Split | Percentage | Purpose |
| --- | --- | --- |
| Training | 70% | Model learning |
| Validation | 20% | Hyperparameter tuning |
| Test | 10% | Final evaluation |

## Arabic Character Dataset (OCR)

A separate dataset was prepared for the Arabic OCR model:

### Supported Characters

Arabic Letters (16):

| Latin | Arabic | Name |
| --- | --- | --- |
| a | أ | Alef |
| b | ب | Ba |
| g | ج | Jeem |
| d | د | Dal |
| r | ر | Ra |
| s | س | Seen |
| ss | ص | Sad |
| tt | ط | Ta |
| aa | ع | Ain |
| f | ف | Fa |
| kk | ق | Qaf |
| l | ل | Lam |
| m | م | Meem |
| n | ن | Noon |
| 00 | ه | Ha |
| w | و | Waw |
| y | ى | Ya |

Arabic Numerals (10):

| Western | Arabic |
| --- | --- |
| 0 | ٠ |
| 1 | ١ |
| 2 | ٢ |
| 3 | ٣ |
| 4 | ٤ |
| 5 | ٥ |
| 6 | ٦ |
| 7 | ٧ |
| 8 | ٨ |
| 9 | ٩ |

## Data Quality Considerations

### Challenges Addressed

Blur: Motion blur from moving vehicles

Occlusion: Partial plate visibility

Lighting: Shadows and reflections

Angle: Perspective distortion

Distance: Resolution variation

### Quality Metrics

All images manually reviewed

Minimum resolution requirements enforced

Corrupt or unusable images removed

Annotation consistency verified

[IMAGE PLACEHOLDER]
Quality Examples

Caption: Examples of challenging images in the dataset - blur, occlusion, and lighting variations

# Model Training

## Overview

The Egyptian License Plate Recognition system uses two custom-trained YOLOv10 models:

License Plate Detection Model: Locates the plate region within an image

Arabic OCR Model: Recognizes individual characters on the cropped plate

## Model Architecture Selection

### Why YOLOv10?

YOLOv10 was selected for this project due to:

Real-time Performance: Optimized for fast inference suitable for edge deployment

Accuracy: State-of-the-art detection accuracy for object detection tasks

Lightweight Variants: Nano (n) variant suitable for resource-constrained environments

End-to-end Design: NMS-free architecture reduces post-processing complexity

### Model Variant: YOLOv10n (Nano)

| Specification | Value |
| --- | --- |
| Parameters | ~2.3M |
| FLOPs | ~8.2G |
| Input Size | 640×640 |
| Suitable For | Edge devices, real-time applications |

## Training Configuration

### Common Training Parameters

Epochs: 150 (Versions 1 & 2), 70 (Version 3)
Batch Size: 8
Input Resolution: 640×640
Patience: 10 (early stopping)
Optimizer: AdamW (default)

### Augmentation Override

All built-in augmentations were disabled during training to rely solely on dataset-level augmentation:

| Parameter | Value |
| --- | --- |
| HSV Hue/Sat/Val | 0 |
| Rotation | 0 |
| Translation | 0 |
| Scale | 0 |
| Shear | 0 |
| Perspective | 0 |
| Flip (LR/UD) | 0 |
| Mosaic | 0 |
| Mixup | 0 |
| Copy-paste | 0 |

This approach ensured consistent comparison between the three dataset augmentation strategies.

## Model Versions Comparison

Three models were trained using different dataset augmentation versions:

### Performance Results

| Model Version | Precision | Recall | mAP@50 | mAP@50-95 |
| --- | --- | --- | --- | --- |
| Noise-Blur-Crop | 0.9588 | 0.9398 | 0.9750 | 0.7494 |
| Rotation-Shear-Noise | 0.9396 | 0.9253 | 0.9664 | 0.7273 |
| Full Augmentation | 0.9214 | 0.9081 | 0.9565 | 0.5966 |

[IMAGE PLACEHOLDER]
Training Comparison Chart

Caption: Performance metrics comparison across the three model versions

### Best Model Selection

Winner: Noise-Blur-Crop (Version 1)

This model achieved the highest scores across all metrics:

Highest precision (95.88%)

Highest recall (93.98%)

Highest mAP@50 (97.50%)

Highest mAP@50-95 (74.94%)

The simpler augmentation strategy (noise, blur, crop) outperformed more aggressive augmentations, suggesting that excessive transformations may have introduced too much variation for the relatively small dataset.

## Training Curves

[IMAGE PLACEHOLDER]
Loss Curves

Caption: Training and validation loss curves for the best performing model

[IMAGE PLACEHOLDER]
mAP Curves

Caption: mAP@50 and mAP@50-95 progression during training

## Arabic OCR Model

### Purpose

The OCR model detects and classifies individual characters within the cropped license plate image.

### Character Classes

The model recognizes 26 classes (16 Arabic letters + 10 Arabic numerals):

#### Arabic Letters

| Class | Arabic | Unicode |
| --- | --- | --- |
| a | أ | U+0623 |
| b | ب | U+0628 |
| g | ج | U+062C |
| d | د | U+062F |
| r | ر | U+0631 |
| s | س | U+0633 |
| ss | ص | U+0635 |
| tt | ط | U+0637 |
| aa | ع | U+0639 |
| f | ف | U+0641 |
| kk | ق | U+0642 |
| l | ل | U+0644 |
| m | م | U+0645 |
| n | ن | U+0646 |
| 00 | ه | U+0647 |
| w | و | U+0648 |
| y | ى | U+0649 |

#### Arabic Numerals

| Class | Arabic | Unicode |
| --- | --- | --- |
| 0 | ٠ | U+0660 |
| 1 | ١ | U+0661 |
| 2 | ٢ | U+0662 |
| 3 | ٣ | U+0663 |
| 4 | ٤ | U+0664 |
| 5 | ٥ | U+0665 |
| 6 | ٦ | U+0666 |
| 7 | ٧ | U+0667 |
| 8 | ٨ | U+0668 |
| 9 | ٩ | U+0669 |

### Character Extraction Process

Crop license plate region from detection model

Resize to 640×640 for OCR model input

Detect individual character bounding boxes

Sort characters right-to-left (Arabic reading direction)

Map Latin predictions to Arabic Unicode

Validate 6-7 character requirement

[IMAGE PLACEHOLDER]
OCR Detection Example

Caption: OCR model output showing detected character bounding boxes with Arabic labels

## Model Validation

### Egyptian Plate Format Validation

The system enforces Egyptian license plate standards:

Minimum Characters: 6

Maximum Characters: 7

Format: Letters followed by numbers

Detections not meeting these criteria are flagged as invalid.

### Confidence Scoring

The overall plate confidence is calculated as the mean confidence of all detected characters:

Total Confidence = Average(char1_conf, char2_conf, ..., charN_conf)

## Detection Pipeline

The complete detection pipeline operates in three stages:

Input Image
    │
    ▼
┌─────────────────────────┐
│  Stage 1: Plate Detection │
│  Model: yolov10_license_plate_detection.pt │
│  Output: Plate bounding box │
└─────────────────────────┘
    │
    ▼
┌─────────────────────────┐
│  Stage 2: Crop & Resize  │
│  Crop plate region       │
│  Resize to 640×640       │
└─────────────────────────┘
    │
    ▼
┌─────────────────────────┐
│  Stage 3: OCR            │
│  Model: yolov10_Arabic_OCR.pt │
│  Output: Character detections │
└─────────────────────────┘
    │
    ▼
┌─────────────────────────┐
│  Post-processing         │
│  Sort RTL, map to Arabic │
│  Validate format         │
└─────────────────────────┘
    │
    ▼
Final Output: Arabic plate text + confidence

## Model Files

| File | Purpose | Size |
| --- | --- | --- |
| `yolov10_license_plate_detection.pt` | Plate region detection | ~5MB |
| `yolov10_Arabic_OCR.pt` | Character recognition | ~5MB |

## Sample Detection Results

[IMAGE PLACEHOLDER]
Detection Sample 1

Caption: Successful detection showing plate localization and character recognition

[IMAGE PLACEHOLDER]
Detection Sample 2

Caption: Detection under challenging lighting conditions

[IMAGE PLACEHOLDER]
Detection Sample 3

Caption: Detection with annotated character bounding boxes

# Backend API

## Overview

The backend is built with FastAPI, a modern Python web framework optimized for building APIs. It handles image processing, license plate recognition, database storage, and serves data to the dashboard.

Production URL: https://robotics-egyptian-lpr-production.up.railway.app

## Architecture

[IMAGE PLACEHOLDER]
Backend Architecture

Caption: FastAPI backend architecture showing component interactions

### Core Components

| Component | Responsibility |
| --- | --- |
| FastAPI App | HTTP request handling, routing |
| LPR Service | License plate detection and OCR |
| Database Module | PostgreSQL async operations |
| Cloudinary Helper | Image upload to CDN |

## API Endpoints

### Health Check

GET /

Returns server status confirmation.

Response:

{
  "status": "ok",
  "message": "Egyptian License Plate Recognition API"
}

### License Plate Recognition

POST /api/recognize

Processes an uploaded image and returns detected license plate information.

Request: Multipart form data with image file

Response:

{
  "success": true,
  "plate": "أ ب ج ١ ٢ ٣",
  "confidence": 0.95,
  "image_url": "https://cloudinary.com/...",
  "error": null
}

Processing Flow:

Receive image as multipart form data

Decode image to numpy array

Run through LPR detection pipeline

Upload car image and plate crop to Cloudinary

Store detection record in PostgreSQL

Return results to client

### Dashboard Data

GET /api/dashboard

Retrieves all detection records for dashboard display.

Response:

{
  "vehicles_today": 42,
  "last_detection": "2024-12-24T15:30:45.123Z",
  "detections": [
    {
      "id": 1,
      "car_image": "https://cloudinary.com/car_123.jpg",
      "lp_image": "https://cloudinary.com/plate_123.jpg",
      "lp_number": "أ ب ج ١ ٢ ٣",
      "confidence": 0.95,
      "created_at": "2024-12-24T15:30:45.123Z"
    }
  ]
}

## License Plate Recognition Service

The LPR service implements the three-stage detection pipeline.

### Detection Pipeline

Image Input
    │
    ▼
┌────────────────────────────┐
│  1. License Plate Detection │
│  YOLOv10 model detects      │
│  plate bounding box         │
└────────────────────────────┘
    │
    ▼
┌────────────────────────────┐
│  2. Crop & Preprocess       │
│  Extract plate region       │
│  Resize to 640×640          │
└────────────────────────────┘
    │
    ▼
┌────────────────────────────┐
│  3. Arabic OCR              │
│  Detect individual chars    │
│  Map to Arabic Unicode      │
│  Sort right-to-left         │
└────────────────────────────┘
    │
    ▼
Output: Plate text + confidence

### Arabic Character Mapping

The OCR model outputs Latin class names that are mapped to Arabic Unicode:

ARABIC_MAP = {
    'a': 'أ', 'b': 'ب', 'g': 'ج', 'd': 'د',
    'r': 'ر', 's': 'س', 'ss': 'ص', 'tt': 'ط',
    'aa': 'ع', 'f': 'ف', 'kk': 'ق', 'l': 'ل',
    'm': 'م', 'n': 'ن', '00': 'ه', 'w': 'و', 'y': 'ى',
    '0': '٠', '1': '١', '2': '٢', '3': '٣', '4': '٤',
    '5': '٥', '6': '٦', '7': '٧', '8': '٨', '9': '٩'
}

### Plate Validation

Egyptian plates must contain 6-7 characters. Detections outside this range are flagged as invalid.

## Database Integration

### PostgreSQL with Neon

The system uses Neon's serverless PostgreSQL for persistent storage.

### Detection Record Schema

| Field | Type | Description |
| --- | --- | --- |
| `id` | Integer | Primary key |
| `car_image` | String | Cloudinary URL of vehicle image |
| `lp_image` | String | Cloudinary URL of plate crop |
| `lp_number` | String | Detected Arabic plate text |
| `confidence` | Float | Detection confidence (0-1) |
| `created_at` | DateTime | Timestamp (Egypt timezone UTC+2) |

### Database Operations

Insert Detection: Store new plate recognition results

Get All Detections: Retrieve records ordered by newest first

Count Today's Vehicles: Count detections from current day

Get Last Detection Time: Retrieve most recent timestamp

## Image Storage

### Cloudinary CDN

Processed images are uploaded to Cloudinary for persistent storage:

| Folder | Content |
| --- | --- |
| `lpr/cars` | Full vehicle images |
| `lpr/plates` | Cropped plate images with annotations |

Benefits:

Global CDN for fast image delivery

Automatic image optimization

Secure HTTPS URLs

No local storage requirements

## Dependencies

| Package | Purpose |
| --- | --- |
| `fastapi` | Web framework |
| `uvicorn` | ASGI server |
| `python-multipart` | Form data parsing |
| `ultralytics` | YOLOv10 models |
| `opencv-python-headless` | Image processing |
| `numpy` | Array operations |
| `supervision` | Detection utilities |
| `asyncpg` | Async PostgreSQL driver |
| `databases` | Database abstraction |
| `cloudinary` | Image upload SDK |
| `python-dotenv` | Environment variables |

## Environment Variables

| Variable | Description |
| --- | --- |
| `DATABASE_URL` | PostgreSQL connection string |
| `CLOUDINARY_CLOUD_NAME` | Cloudinary account name |
| `CLOUDINARY_API_KEY` | Cloudinary API key |
| `CLOUDINARY_API_SECRET` | Cloudinary API secret |
| `PORT` | Server port (default: 8000) |

## Request Flow Diagram

[IMAGE PLACEHOLDER]
API Request Flow

Caption: Complete request flow from image upload to response

ESP32-CAM                    FastAPI Backend
    │                              │
    │  POST /api/recognize         │
    │  [image file]                │
    │─────────────────────────────▶│
    │                              │
    │                     ┌────────┴────────┐
    │                     │ Decode Image    │
    │                     │ Run Detection   │
    │                     │ Run OCR         │
    │                     │ Upload Images   │
    │                     │ Store in DB     │
    │                     └────────┬────────┘
    │                              │
    │  Response JSON               │
    │  {success, plate, conf}      │
    │◀─────────────────────────────│
    │                              │

# Dashboard

## Overview

The monitoring dashboard is a React-based web application that displays real-time license plate detection results. It features a dark cyberpunk-themed design optimized for continuous monitoring scenarios.

[IMAGE PLACEHOLDER]
Dashboard Screenshot

Caption: Full dashboard view showing detection cards and header statistics

## Technology Stack

| Technology | Purpose |
| --- | --- |
| React 18 | UI framework |
| TypeScript | Type-safe JavaScript |
| Vite | Build tool and dev server |
| Tailwind CSS | Utility-first styling |
| Radix UI | Headless component primitives |
| Lucide React | Icon library |

## Design System

### Color Palette

The dashboard uses a dark theme with cyan accents:

| Variable | Hex Code | Usage |
| --- | --- | --- |
| `--bg-primary` | #121212 | Main background |
| `--bg-secondary` | #1e1e1e | Secondary areas |
| `--bg-card` | #242424 | Card backgrounds |
| `--accent-cyan` | #00FFFF | Primary accent |
| `--accent-green` | #00FF41 | Status indicators |
| `--text-primary` | #e0e0e0 | Main text |
| `--text-secondary` | #a0a0a0 | Secondary text |
| `--border-color` | #333333 | Borders |

[IMAGE PLACEHOLDER]
Color Palette

Caption: Dashboard color scheme visualization

### Typography

Body Font: Inter (system fallback)

License Plate: Roboto Mono (monospace for authentic plate rendering)

Character Spacing: 0.1em on plate numbers

## Layout Structure

### Header Section

[IMAGE PLACEHOLDER]
Dashboard Header

Caption: Dashboard header with live status and statistics

The sticky header contains:

Title: "Egyptian Plate Recognition"

Live Status Indicator: Pulsing green dot with "Live Monitor - Active"

Today's Count: Number of vehicles detected today

Last Detection: Relative time since last detection (e.g., "5m ago")

### Detection Grid

The main content area displays detection cards in a responsive grid:

| Breakpoint | Columns |
| --- | --- |
| Mobile | 1 column |
| Small (sm) | 2 columns |
| Large (lg) | 3 columns |
| Extra Large (xl) | 4 columns |

## Components

### Detection Card

Each detected vehicle is displayed in a card containing:

[IMAGE PLACEHOLDER]
Detection Card

Caption: Anatomy of a detection card showing all elements

Vehicle Image: Full captured image (16:9 aspect ratio)

Plate Thumbnail: Small cropped plate image

Plate Number: Arabic text in white box with monospace font

Timestamp: Detection time (HH:MM:SS format)

Confidence Badge:

Green checkmark + green text if ≥90%

Orange text if <90%

### Card Interaction

Hover Effect: Border transitions from #333 to cyan (#00FFFF)

Image Fallback: SVG placeholder if image fails to load

## Real-Time Updates

### Auto-Refresh Mechanism

The dashboard automatically fetches new data from the API:

Interval: Every 30 seconds

Endpoint: GET /api/dashboard

Behavior: Seamlessly updates without page reload

### Relative Time Display

Timestamps are displayed as human-readable relative times:

| Time Elapsed | Display |
| --- | --- |
| < 1 minute | "Just now" |
| 1-59 minutes | "Xm ago" |
| 1-23 hours | "Xh ago" |
| 1+ days | "Xd ago" |

## Data Flow

┌─────────────────────────────┐
│      Dashboard (React)       │
└──────────────┬──────────────┘
               │
               │ GET /api/dashboard
               │ (every 30 seconds)
               ▼
┌─────────────────────────────┐
│      FastAPI Backend         │
└──────────────┬──────────────┘
               │
               │ Query
               ▼
┌─────────────────────────────┐
│    PostgreSQL Database       │
└──────────────┬──────────────┘
               │
               │ Response
               ▼
┌─────────────────────────────┐
│   Dashboard State Update     │
│   - vehicles_today           │
│   - last_detection           │
│   - detections[]             │
└─────────────────────────────┘

## State Management

### Dashboard Data Interface

interface Detection {
  id: number;
  car_image: string;      // Cloudinary URL
  lp_image: string;       // Cloudinary URL
  lp_number: string;      // Arabic plate text
  confidence: number;     // 0-1 decimal
  created_at: string;     // ISO timestamp
}

interface DashboardData {
  vehicles_today: number;
  last_detection: string | null;
  detections: Detection[];
}

### Application States

| State | Display |
| --- | --- |
| Loading | "Loading detections..." spinner |
| Empty | "No detections yet. Waiting for vehicles..." |
| Error | Error message with retry button |
| Active | Grid of detection cards |

## Responsive Design

[IMAGE PLACEHOLDER]
Responsive Views

Caption: Dashboard appearance across different screen sizes

### Mobile Optimizations

Single-column card layout

Reduced padding and margins

Touch-friendly tap targets

Optimized image loading

### Desktop Features

Multi-column grid layout

Hover interactions

Maximum container width (1800px)

## UI Components Library

The dashboard utilizes shadcn/ui components built on Radix UI:

| Component | Usage |
| --- | --- |
| Cards | Detection result containers |
| Buttons | Retry action |
| Icons | Checkmark, status indicators |
| Skeleton | Loading placeholders |

## Performance Considerations

Image Optimization: Images served via Cloudinary CDN

Lazy Loading: Images load as they enter viewport

Error Boundaries: Graceful fallbacks for failed image loads

Efficient Re-renders: React state updates only changed elements

# Hardware Setup

## Overview

The hardware system uses an ESP32-CAM microcontroller with an ultrasonic sensor to automatically detect approaching vehicles and capture license plate images. This creates a compact, portable demonstration unit for the recognition system.

[IMAGE PLACEHOLDER]
Hardware Assembly

Caption: Complete assembled hardware showing ESP32-CAM, ultrasonic sensor, and power supply

## Components List

| Component | Model/Specification | Quantity |
| --- | --- | --- |
| Microcontroller | ESP32-CAM (AI-Thinker) | 1 |
| Camera Module | OV2640 (included with ESP32-CAM) | 1 |
| Ultrasonic Sensor | HC-SR04 | 1 |
| Voltage Regulator | 7805 (5V output) | 1 |
| Batteries | 18650 Li-ion (3.7V each) | 2 |
| Battery Holder | 2-cell 18650 holder | 1 |
| Resistors | 1kΩ (1/4W) | 3 |
| FTDI Programmer | USB-to-Serial (for programming) | 1 |
| Breadboard | Standard size | 1 |
| Jumper Wires | Male-to-male, male-to-female | Various |

## Wiring Diagram

[IMAGE PLACEHOLDER]
Wiring Diagram

Caption: Complete wiring diagram showing all connections between components

## GPIO Pin Assignments

### ESP32-CAM Pinout

| GPIO | Function | Connection |
| --- | --- | --- |
| GPIO 13 | TRIG Output | HC-SR04 TRIG pin |
| GPIO 12 | ECHO Input | HC-SR04 ECHO (via voltage divider) |
| GPIO 4 | Flash LED | Built-in (PWM controlled) |
| GPIO 0 | Boot Mode | GND for programming |
| U0R | Serial RX | FTDI TX |
| U0T | Serial TX | FTDI RX |
| 5V | Power Input | 7805 output |
| GND | Ground | Common ground |

### Camera Pins (Internal to ESP32-CAM)

| Pin | GPIO |
| --- | --- |
| PWDN | 32 |
| RESET | -1 (not used) |
| XCLK | 0 |
| SIOD | 26 |
| SIOC | 27 |
| Y9-Y2 | 35, 34, 39, 36, 21, 19, 18, 5 |
| VSYNC | 25 |
| HREF | 23 |
| PCLK | 22 |

## Power Supply

### Battery Configuration

Two 18650 batteries connected in series provide approximately 7.4V (nominal):

Battery 1 (+) ─────┐
    3.7V           │
Battery 1 (-) ─────┼───── Battery 2 (+)
                   │          3.7V
                   └───── Battery 2 (-)
                              │
                         7.4V Total

### Voltage Regulator (7805)

The 7805 regulates the battery voltage down to stable 5V:

| Pin | Name | Connection |
| --- | --- | --- |
| 1 | Input | Battery positive (~7.4V) |
| 2 | Ground | Common ground |
| 3 | Output | 5V to ESP32-CAM and HC-SR04 |

[IMAGE PLACEHOLDER]
7805 Pinout

Caption: 7805 voltage regulator pinout (front view)

Note: The 7805 requires at least 7V input for stable 5V output. Ensure batteries are charged above 3.5V each.

## Ultrasonic Sensor (HC-SR04)

### Specifications

| Parameter | Value |
| --- | --- |
| Operating Voltage | 5V DC |
| Operating Current | 15mA |
| Measuring Range | 2cm - 400cm |
| Resolution | 0.3cm |
| Trigger Input | 10µs TTL pulse |
| Echo Output | 5V TTL signal |

### Connections

| HC-SR04 Pin | Connection |
| --- | --- |
| VCC | 5V rail |
| TRIG | GPIO 13 (direct) |
| ECHO | GPIO 12 (via voltage divider) |
| GND | Common ground |

## Voltage Divider Circuit

Critical: The HC-SR04 ECHO pin outputs 5V, but ESP32 GPIO pins are 3.3V tolerant. A voltage divider is required to protect the ESP32.

### Circuit Design

ECHO (5V) ──────┬─────[1kΩ]─────┬─────[1kΩ]────[1kΩ]───── GND
                │               │
                │               └──────► GPIO 12 (3.3V)
                │
           HC-SR04 ECHO

### Voltage Calculation

Using the voltage divider formula:

V_out = V_in × (R2 / (R1 + R2))
V_out = 5V × (2kΩ / (1kΩ + 2kΩ))
V_out = 5V × (2/3)
V_out = 3.33V ✓ (safe for ESP32)

Where:

R1 = 1kΩ (between ECHO and GPIO)

R2 = 2kΩ (two 1kΩ in series, between GPIO and GND)

[IMAGE PLACEHOLDER]
Voltage Divider

Caption: Voltage divider circuit for ECHO pin protection

## Detection Logic

### Trigger Parameters

| Parameter | Value | Purpose |
| --- | --- | --- |
| Trigger Distance | 15 cm | Maximum distance to trigger capture |
| Minimum Distance | 2 cm | Ignore very close readings (noise) |
| Debounce Delay | 3000 ms | Wait between captures |

### Distance Calculation

The ultrasonic sensor measures time-of-flight:

Distance = (Duration × Speed of Sound) / 2
Distance = (Duration × 0.034 cm/µs) / 2

Speed of sound = 343 m/s = 0.0343 cm/µs

### Multi-Sample Averaging

Three readings are taken and averaged for stability:

for (int i = 0; i < 3; i++) {
    long dist = measureDistance();
    if (dist > 0 && dist < 400) {
        totalDistance += dist;
        validReadings++;
    }
    delay(10);
}
long avgDistance = totalDistance / validReadings;

## Camera Configuration

### Resolution Settings

| Condition | Resolution | Quality | Frame Buffers |
| --- | --- | --- | --- |
| With PSRAM | SVGA (800×600) | 10 | 2 |
| Without PSRAM | VGA (640×480) | 12 | 1 |

### Flash LED Control

The built-in flash LED (GPIO 4) supports PWM brightness control:

| Brightness | PWM Value | Description |
| --- | --- | --- |
| Off | 0 | No flash |
| 25% | 64 | Dim |
| 50% | 128 | Medium |
| 75% | 192 | Bright |
| 100% | 255 | Maximum |

## Programming Mode

### FTDI Connection

To program the ESP32-CAM, connect an FTDI USB-to-Serial adapter:

| FTDI Pin | ESP32-CAM Pin |
| --- | --- |
| TX | U0R (RX) |
| RX | U0T (TX) |
| GND | GND |
| VCC | Not connected* |

*ESP32-CAM is powered by batteries/7805, not FTDI

### Boot Mode

For programming, GPIO 0 must be held LOW:

Connect GPIO 0 to GND

Press RST button or power cycle

Upload code via Arduino IDE

Disconnect GPIO 0 from GND

Press RST to run program

## Camera Stream Server

The ESP32 runs a web server for live preview:

URL: http://<ESP32-IP>/stream

Port: 80

Format: MJPEG stream

This allows real-time viewing of the camera feed for debugging and alignment.

## WiFi Configuration

### Connection Parameters

const char* ssid = "YOUR_WIFI_SSID";
const char* password = "YOUR_WIFI_PASSWORD";

### Auto-Reconnection

If WiFi disconnects, the system automatically attempts reconnection before each capture.

## Image Transmission

When an object is detected within range:

Flash LED turns on (100ms warm-up)

Camera captures JPEG image

Flash LED turns off

Image sent via HTTPS POST to API

Response parsed for plate number and confidence

### API Request

POST https://robotics-egyptian-lpr-production.up.railway.app/api/recognize
Content-Type: multipart/form-data

[JPEG image data]

## Assembly Photos

[IMAGE PLACEHOLDER]
Breadboard Layout

Caption: Recommended breadboard component layout

[IMAGE PLACEHOLDER]
Final Assembly

Caption: Completed prototype ready for testing

[IMAGE PLACEHOLDER]
Maket Setup

Caption: Demonstration setup with toy car and Egyptian license plate

# Deployment

## Overview

The Egyptian License Plate Recognition system is deployed across multiple cloud services for production operation:

| Service | Platform | Purpose |
| --- | --- | --- |
| Backend API | Railway | Image processing and ML inference |
| Dashboard | Railway | Real-time monitoring interface |
| Database | Neon | PostgreSQL data storage |
| Image Storage | Cloudinary | CDN for captured images |

[IMAGE PLACEHOLDER]
Deployment Architecture

Caption: Cloud deployment architecture showing service connections

## Railway Platform

Railway provides container-based hosting with automatic deployments from Git repositories.

### API Service

URL: https://robotics-egyptian-lpr-production.up.railway.app

#### Container Configuration

The API runs in a Docker container with the following setup:

FROM python:3.11-slim

# Install system dependencies for OpenCV
RUN apt-get update && apt-get install -y \
    libgl1 \
    libglib2.0-0

# Install Python packages
COPY requirements.txt .
RUN pip install --no-cache-dir -r requirements.txt

# Copy application code and ML models
COPY api/ ./api/
COPY yolov10_license_plate_detection.pt .
COPY yolov10_Arabic_OCR.pt .

# Start server
CMD uvicorn main:app --host 0.0.0.0 --port $PORT

#### Key Features

Auto-scaling: Handles variable traffic loads

SSL/TLS: Automatic HTTPS certificates

Environment Variables: Secure secrets management

Logging: Built-in request logging and monitoring

### Dashboard Service

The React dashboard is deployed as a static site:

Build Command: npm run build

Output Directory: build/

Framework: Vite static site generation

## Neon PostgreSQL

Neon provides serverless PostgreSQL with:

Auto-suspend: Database sleeps when inactive (cost savings)

Auto-scaling: Compute scales with demand

Branching: Database branches for development

Connection Pooling: Efficient connection management

### Database Schema

CREATE TABLE detections (
    id SERIAL PRIMARY KEY,
    car_image VARCHAR(500),
    lp_image VARCHAR(500),
    lp_number VARCHAR(50),
    confidence FLOAT,
    created_at TIMESTAMP WITH TIME ZONE DEFAULT NOW()
);

### Connection

The async PostgreSQL driver connects using:

DATABASE_URL=postgresql://user:password@host:port/database?sslmode=require

## Cloudinary CDN

Cloudinary handles image storage and delivery:

### Folder Structure

| Folder | Content |
| --- | --- |
| `lpr/cars` | Full vehicle images |
| `lpr/plates` | Annotated plate crops |

### Image Processing

Automatic format optimization

Responsive delivery

Global CDN distribution

Secure signed URLs

### Upload Flow

ESP32 captures image
        │
        ▼
FastAPI receives image
        │
        ▼
Process with YOLOv10
        │
        ▼
Upload to Cloudinary
        │
        ▼
Store URL in database

## Environment Configuration

### Required Variables

| Variable | Service | Description |
| --- | --- | --- |
| `DATABASE_URL` | Neon | PostgreSQL connection string |
| `CLOUDINARY_CLOUD_NAME` | Cloudinary | Account identifier |
| `CLOUDINARY_API_KEY` | Cloudinary | API authentication |
| `CLOUDINARY_API_SECRET` | Cloudinary | API secret key |
| `PORT` | Railway | Server port (auto-set) |
| `VITE_API_URL` | Dashboard | Backend API URL |

### Security Notes

All secrets stored as Railway environment variables

Database connections use SSL/TLS

Cloudinary uploads use signed requests

CORS configured for dashboard domain

## Service Communication

### Data Flow

┌──────────────┐     HTTPS      ┌──────────────┐
│   ESP32-CAM   │───────────────▶│  Railway API  │
└──────────────┘                 └──────┬───────┘
                                        │
                    ┌───────────────────┼───────────────────┐
                    │                   │                   │
                    ▼                   ▼                   ▼
            ┌──────────────┐   ┌──────────────┐   ┌──────────────┐
            │   Cloudinary  │   │     Neon     │   │   Dashboard  │
            │   (Images)    │   │ (PostgreSQL) │   │   (React)    │
            └──────────────┘   └──────────────┘   └──────────────┘

### API Endpoints

| Endpoint | Method | Source | Purpose |
| --- | --- | --- | --- |
| `/api/recognize` | POST | ESP32 | Upload image for recognition |
| `/api/dashboard` | GET | Dashboard | Fetch detection records |

## Scaling Considerations

### Current Limits

| Resource | Limit |
| --- | --- |
| API Memory | 512MB |
| Request Timeout | 60 seconds |
| Database Connections | 100 pooled |
| Image Size | 10MB max |

### Performance Optimizations

Model Lazy Loading: ML models loaded on first request

Connection Pooling: Reuse database connections

Image Compression: JPEG quality optimization

CDN Caching: Static assets cached globally

## Monitoring

### Railway Dashboard

Request logs and metrics

Memory and CPU usage

Deployment history

Environment variable management

### Health Checks

The API provides a health endpoint:

GET /
Response: {"status": "ok", "message": "..."}

## Cost Structure

| Service | Pricing Model |
| --- | --- |
| Railway | Usage-based (memory × time) |
| Neon | Free tier available, then usage-based |
| Cloudinary | Free tier (25GB), then storage + bandwidth |

## Deployment Diagram

[IMAGE PLACEHOLDER]
Full Deployment

Caption: Complete deployment architecture with all services and connections

# System Architecture

## Overview

The Egyptian License Plate Recognition system is a complete IoT-to-Cloud solution that combines edge hardware, machine learning, cloud services, and real-time visualization. This document describes the complete system architecture and data flow.

[IMAGE PLACEHOLDER]
System Architecture Overview

Caption: High-level system architecture showing all components and their interactions

## Architecture Layers

### Layer 1: Edge Hardware (IoT)

The physical detection and capture system:

┌─────────────────────────────────────────────┐
│              HARDWARE LAYER                  │
│                                             │
│  ┌─────────────┐      ┌─────────────┐       │
│  │ HC-SR04     │      │ ESP32-CAM   │       │
│  │ Ultrasonic  │─────▶│ + OV2640    │       │
│  │ Sensor      │      │ Camera      │       │
│  └─────────────┘      └──────┬──────┘       │
│                              │              │
│                      ┌───────▼───────┐      │
│                      │ 7805 Voltage  │      │
│                      │ Regulator     │      │
│                      └───────┬───────┘      │
│                              │              │
│                      ┌───────▼───────┐      │
│                      │ 2x 18650      │      │
│                      │ Batteries     │      │
│                      └───────────────┘      │
└─────────────────────────────────────────────┘

Functions:

Vehicle proximity detection (ultrasonic)

Image capture (camera)

WiFi transmission

Local preview stream

### Layer 2: Cloud Backend (ML Processing)

The machine learning inference and API layer:

┌─────────────────────────────────────────────┐
│              BACKEND LAYER                   │
│                                             │
│  ┌─────────────────────────────────────┐    │
│  │          FastAPI Server              │    │
│  │                                      │    │
│  │  ┌────────────┐  ┌────────────┐     │    │
│  │  │ Plate      │  │ Arabic     │     │    │
│  │  │ Detection  │  │ OCR        │     │    │
│  │  │ YOLOv10    │  │ YOLOv10    │     │    │
│  │  └─────┬──────┘  └──────┬─────┘     │    │
│  │        │                │           │    │
│  │        └───────┬────────┘           │    │
│  │                │                    │    │
│  │        ┌───────▼───────┐            │    │
│  │        │  LPR Service  │            │    │
│  │        └───────────────┘            │    │
│  └─────────────────────────────────────┘    │
│                                             │
└─────────────────────────────────────────────┘

Functions:

Image reception via REST API

License plate detection

Arabic character recognition

Confidence scoring

### Layer 3: Data Persistence

Storage and database layer:

┌─────────────────────────────────────────────┐
│              STORAGE LAYER                   │
│                                             │
│  ┌──────────────────┐  ┌──────────────────┐ │
│  │    Cloudinary    │  │       Neon       │ │
│  │                  │  │                  │ │
│  │  ┌────────────┐  │  │  ┌────────────┐  │ │
│  │  │ Car Images │  │  │  │ Detections │  │ │
│  │  └────────────┘  │  │  │   Table    │  │ │
│  │  ┌────────────┐  │  │  └────────────┘  │ │
│  │  │Plate Crops │  │  │                  │ │
│  │  └────────────┘  │  │  PostgreSQL      │ │
│  │                  │  │                  │ │
│  │  CDN Delivery    │  │  Serverless      │ │
│  └──────────────────┘  └──────────────────┘ │
│                                             │
└─────────────────────────────────────────────┘

Functions:

Image hosting (Cloudinary CDN)

Detection records (PostgreSQL)

Async database operations

### Layer 4: Presentation (Dashboard)

User interface layer:

┌─────────────────────────────────────────────┐
│            PRESENTATION LAYER                │
│                                             │
│  ┌─────────────────────────────────────┐    │
│  │         React Dashboard              │    │
│  │                                      │    │
│  │  ┌──────────┐  ┌──────────────────┐ │    │
│  │  │ Header   │  │ Detection Grid   │ │    │
│  │  │ Stats    │  │                  │ │    │
│  │  └──────────┘  │ ┌────┐ ┌────┐   │ │    │
│  │                │ │Card│ │Card│   │ │    │
│  │                │ └────┘ └────┘   │ │    │
│  │                │ ┌────┐ ┌────┐   │ │    │
│  │                │ │Card│ │Card│   │ │    │
│  │                │ └────┘ └────┘   │ │    │
│  │                └──────────────────┘ │    │
│  └─────────────────────────────────────┘    │
│                                             │
└─────────────────────────────────────────────┘

Functions:

Real-time detection display

Statistics and metrics

Auto-refresh (30 seconds)

## Complete Data Flow

### End-to-End Process

VEHICLE            ESP32-CAM           RAILWAY API          DATABASE         DASHBOARD
    │                   │                    │                   │                │
    │   Approaches      │                    │                   │                │
    │──────────────────▶│                    │                   │                │
    │                   │                    │                   │                │
    │               Ultrasonic               │                   │                │
    │               Detection                │                   │                │
    │                   │                    │                   │                │
    │               Distance < 15cm          │                   │                │
    │                   │                    │                   │                │
    │               Capture Image            │                   │                │
    │                   │                    │                   │                │
    │                   │  POST /recognize   │                   │                │
    │                   │───────────────────▶│                   │                │
    │                   │                    │                   │                │
    │                   │              Detect Plate              │                │
    │                   │              Run OCR                   │                │
    │                   │                    │                   │                │
    │                   │                    │  Upload Images    │                │
    │                   │                    │──────────────────▶│ (Cloudinary)   │
    │                   │                    │                   │                │
    │                   │                    │  Store Record     │                │
    │                   │                    │──────────────────▶│ (PostgreSQL)   │
    │                   │                    │                   │                │
    │                   │  Response          │                   │                │
    │                   │◀───────────────────│                   │                │
    │                   │                    │                   │                │
    │                   │                    │                   │ GET /dashboard │
    │                   │                    │                   │◀───────────────│
    │                   │                    │                   │                │
    │                   │                    │                   │  Return Data   │
    │                   │                    │                   │───────────────▶│
    │                   │                    │                   │                │
    │                   │                    │                   │          Display
    │                   │                    │                   │          Detection
    │                   │                    │                   │                │

## Detection Pipeline Detail

### Three-Stage ML Processing

Input Image (from ESP32)
         │
         ▼
┌────────────────────────┐
│   STAGE 1: Detection   │
│                        │
│   YOLOv10 License      │
│   Plate Detection      │
│                        │
│   Output: Bounding box │
│   of plate region      │
└───────────┬────────────┘
            │
            ▼
┌────────────────────────┐
│   STAGE 2: Crop        │
│                        │
│   Extract plate region │
│   Resize to 640×640    │
│                        │
│   Output: Plate crop   │
└───────────┬────────────┘
            │
            ▼
┌────────────────────────┐
│   STAGE 3: OCR         │
│                        │
│   YOLOv10 Arabic OCR   │
│   Detect characters    │
│   Sort right-to-left   │
│   Map to Arabic        │
│                        │
│   Output: Arabic text  │
│   + confidence score   │
└───────────┬────────────┘
            │
            ▼
    Final Result
    "أ ب ج ١ ٢ ٣"
    Confidence: 95.2%

## Technology Integration

### Communication Protocols

| Connection | Protocol | Security |
| --- | --- | --- |
| ESP32 → API | HTTPS | TLS 1.3 |
| API → Database | PostgreSQL | SSL |
| API → Cloudinary | HTTPS | API Key |
| Dashboard → API | HTTPS | CORS |

### Data Formats

| Transfer | Format |
| --- | --- |
| Image Upload | Multipart Form (JPEG) |
| API Response | JSON |
| Database | SQL |
| Dashboard Data | JSON |

## Technology Stack Summary

[IMAGE PLACEHOLDER]
Technology Stack

Caption: Complete technology stack visualization

### By Category

| Category | Technologies |
| --- | --- |
| Hardware | ESP32-CAM, HC-SR04, 7805, 18650 |
| ML/AI | YOLOv10, Ultralytics, OpenCV |
| Backend | FastAPI, Python, Uvicorn |
| Database | PostgreSQL, Neon, AsyncPG |
| Storage | Cloudinary CDN |
| Frontend | React, TypeScript, Vite, Tailwind |
| Deployment | Railway, Docker |
| Annotation | Roboflow |

## System Characteristics

### Performance Metrics

| Metric | Value |
| --- | --- |
| Detection Accuracy | 97.5% mAP@50 |
| OCR Precision | 95.88% |
| End-to-end Latency | ~2-3 seconds |
| Dashboard Refresh | 30 seconds |

### Reliability Features

WiFi auto-reconnection

Database connection pooling

Image fallback handling

Error state recovery

### Scalability Points

Horizontal API scaling (Railway)

Database auto-scaling (Neon)

CDN global distribution (Cloudinary)

Stateless API design

## System Diagram

┌─────────────────────────────────────────────────────────────────────────────┐
│                     EGYPTIAN LICENSE PLATE RECOGNITION                       │
├─────────────────────────────────────────────────────────────────────────────┤
│                                                                             │
│   ┌───────────────┐                                    ┌───────────────┐    │
│   │               │           WiFi/Internet            │               │    │
│   │   ESP32-CAM   │═══════════════════════════════════▶│  Railway API  │    │
│   │ + Ultrasonic  │           POST /recognize          │   (FastAPI)   │    │
│   │               │                                    │               │    │
│   └───────────────┘                                    └───────┬───────┘    │
│          │                                                     │            │
│          │ Camera                                              │            │
│          │ Stream                              ┌───────────────┼───────┐    │
│          ▼                                     │               │       │    │
│   ┌───────────────┐                    ┌───────▼─────┐ ┌───────▼─────┐│    │
│   │  Local Debug  │                    │ Cloudinary  │ │    Neon     ││    │
│   │  http://IP/   │                    │   (CDN)     │ │ PostgreSQL  ││    │
│   └───────────────┘                    └─────────────┘ └─────────────┘│    │
│                                                │               │       │    │
│                                                └───────────────┼───────┘    │
│                                                                │            │
│                                                        ┌───────▼───────┐    │
│                                                        │   Dashboard   │    │
│                                                        │   (React)     │    │
│                                                        │               │    │
│                                                        │ GET /dashboard│    │
│                                                        └───────────────┘    │
│                                                                             │
└─────────────────────────────────────────────────────────────────────────────┘

## Demonstration Setup

The project is demonstrated using a miniature maket (model):

[IMAGE PLACEHOLDER]
Maket Setup

Caption: Demonstration maket showing toy car with Egyptian license plate approaching the sensor

Components:

Toy car with printed Egyptian license plate

ESP32-CAM mounted at plate level

Ultrasonic sensor positioned to detect approaching vehicles

Battery-powered portable operation

This allows showcasing the complete detection flow without requiring actual vehicles.