# Egyptian License Plate Recognition System

Project Documentation Summary

# Egyptian License Plate Recognition System

## Project Overview

This project presents a complete AI solution for automatic Egyptian license plate recognition. The system uses an ESP32-CAM with ultrasonic sensor to detect vehicles, captures images, processes them through a cloud-based YOLOv10 model, and displays results on a real-time dashboard.

[IMAGE: System Overview]

Caption: High-level system architecture

### Problem Statement

Egyptian license plates use Arabic characters and numerals, presenting unique challenges for automated recognition:

Right-to-left reading direction

Arabic-Indic numerals (٠١٢٣٤٥٦٧٨٩)

16 possible Arabic letters specific to Egyptian plates

Variable lighting and environmental conditions

### Solution Overview

The system addresses these challenges through:

Custom-trained YOLOv10 models for detection and OCR

Edge-based triggering with ESP32-CAM

Cloud processing for ML inference

Real-time monitoring dashboard

### Key Achievements

| Metric | Value |
| --- | --- |
| Detection Precision | 95.88% |
| Detection Recall | 93.98% |
| mAP@50 | 97.50% |
| mAP@50-95 | 74.94% |
| Dataset Size | 1,800+ images |

### Technology Stack

| Layer | Technology |
| --- | --- |
| Hardware | ESP32-CAM, HC-SR04 Ultrasonic |
| ML Framework | YOLOv10 (Ultralytics) |
| Backend | FastAPI (Python) |
| Database | PostgreSQL (Neon) |
| Storage | Cloudinary CDN |
| Frontend | React + TypeScript |
| Deployment | Railway |

### Live Deployments

API: https://robotics-egyptian-lpr-production.up.railway.app

Dataset: https://universe.roboflow.com/vguard-vxduz/egyptian-license-plate-detection

## Dataset Preparation

### Dataset Overview

| Metric | Value |
| --- | --- |
| Total Images | 1,800+ |
| Annotation Platform | Roboflow |
| Format | YOLOv10 compatible |
| Classes | License plate region |

Images were manually collected showing Egyptian license plates in various conditions: different lighting, angles, vehicle types, and distances.

### Collection Strategy

The dataset includes images from:

Various vehicle types (cars, trucks, motorcycles)

Different lighting conditions (daylight, shade, evening)

Multiple angles and distances

Urban and highway environments

### Annotation Process

Each image was annotated using Roboflow with bounding boxes around the license plate region. The annotations were exported in YOLOv10-compatible format.

## Model Training

### Model Architecture: YOLOv10n (Nano)

YOLOv10 was selected for this project due to:

Real-time Performance: Optimized for fast inference

Accuracy: State-of-the-art detection accuracy

Lightweight: Nano variant suitable for cloud deployment

NMS-free: End-to-end design reduces complexity

| Specification | Value |
| --- | --- |
| Parameters | ~2.3M |
| FLOPs | ~8.2G |
| Input Size | 640×640 |

### Training Configuration

| Parameter | Value |
| --- | --- |
| Epochs | 150 |
| Batch Size | 8 |
| Input Resolution | 640×640 |
| Patience | 10 (early stopping) |
| Optimizer | AdamW |

### Augmentation Strategy

All built-in YOLOv10 augmentations were disabled. Three dataset-level augmentation versions were tested:

| Augmentation | Parameters |
| --- | --- |
| HSV/Rotation/Scale | 0 (disabled) |
| Flip/Mosaic/Mixup | 0 (disabled) |

### Model Versions Comparison

| Model Version | Precision | Recall | mAP@50 | mAP@50-95 |
| --- | --- | --- | --- | --- |
| Noise-Blur-Crop | 0.9588 | 0.9398 | 0.9750 | 0.7494 |
| Rotation-Shear-Noise | 0.9396 | 0.9253 | 0.9664 | 0.7273 |
| Full Augmentation | 0.9214 | 0.9081 | 0.9565 | 0.5966 |

Best Model: Noise-Blur-Crop achieved highest scores across all metrics. The simpler augmentation strategy outperformed aggressive augmentations, suggesting excessive transformations introduced too much variation for the dataset size.

Caption: Performance metrics comparison across model versions

## Arabic OCR Model

### Character Classes

The OCR model recognizes 26 classes (16 Arabic letters + 10 Arabic numerals):

Arabic Letters Used on Egyptian Plates

| Class | Arabic | Name |
| --- | --- | --- |
| a | أ | Alef |
| b | ب | Ba |
| g | ج | Jeem |
| d | د | Dal |
| r | ر | Ra |
| s | س | Seen |
| ss | ص | Sad |
| tt | ط | Ta |
| aa | ع | Ain |
| f | ف | Fa |
| kk | ق | Qaf |
| l | ل | Lam |
| m | م | Meem |
| n | ن | Noon |
| 00 | ه | Ha |
| w | و | Waw |
| y | ى | Ya |

Arabic-Indic Numerals

| Class | Arabic | Western |
| --- | --- | --- |
| 0 | ٠ | 0 |
| 1 | ١ | 1 |
| 2 | ٢ | 2 |
| 3 | ٣ | 3 |
| 4 | ٤ | 4 |
| 5 | ٥ | 5 |
| 6 | ٦ | 6 |
| 7 | ٧ | 7 |
| 8 | ٨ | 8 |
| 9 | ٩ | 9 |

### Character Extraction Process

Crop license plate region from detection model

Resize to 640×640 for OCR model input

Detect individual character bounding boxes

Sort characters right-to-left (Arabic reading direction)

Map Latin predictions to Arabic Unicode

Validate 6-7 character Egyptian plate format

Egyptian plates follow a standard format with letters on the left side and numbers on the right side.

## Backend API (FastAPI)

### Architecture

The backend is built with FastAPI, a modern Python web framework. It handles image processing, license plate recognition, database storage, and serves data to the dashboard.

Production URL: https://robotics-egyptian-lpr-production.up.railway.app

### Core Components

| Component | Responsibility |
| --- | --- |
| FastAPI App | HTTP request handling, routing |
| LPR Service | License plate detection and OCR |
| Database Module | PostgreSQL async operations |
| Cloudinary Helper | Image upload to CDN |

### Detection Pipeline

The backend processes images through a three-stage pipeline:

Image → Plate Detection → Crop & Resize → Arabic OCR → Result

Stage 1: Plate Detection

YOLOv10 model detects plate bounding box

Model: yolov10_license_plate_detection.pt

Stage 2: Crop & Preprocess

Extract plate region from image

Resize to 640×640 for OCR input

Stage 3: Arabic OCR

Detect individual characters

Map to Arabic Unicode

Sort right-to-left

### API Endpoints

| Endpoint | Method | Purpose |
| --- | --- | --- |
| `/` | GET | Health check |
| `/api/recognize` | POST | Process image, return plate text |
| `/api/dashboard` | GET | Retrieve all detection records |

### Recognition Endpoint

Request: Multipart form data with image file

Response:

{
  "success": true,
  "plate": "أ ب ج ١ ٢ ٣",
  "confidence": 0.95,
  "image_url": "https://cloudinary.com/...",
  "error": null
}

### Dashboard Endpoint

Response:

{
  "vehicles_today": 42,
  "last_detection": "2024-12-24T15:30:45.123Z",
  "detections": [
    {
      "id": 1,
      "car_image": "https://cloudinary.com/car.jpg",
      "lp_image": "https://cloudinary.com/plate.jpg",
      "lp_number": "أ ب ج ١ ٢ ٣",
      "confidence": 0.95,
      "created_at": "2024-12-24T15:30:45.123Z"
    }
  ]
}

### Dependencies

| Package | Purpose |
| --- | --- |
| fastapi | Web framework |
| uvicorn | ASGI server |
| ultralytics | YOLOv10 models |
| opencv-python-headless | Image processing |
| asyncpg | Async PostgreSQL driver |
| cloudinary | Image upload SDK |

## Dashboard (React)

### Technology Stack

Framework: React with TypeScript

Build Tool: Vite

Styling: Tailwind CSS

Theme: Dark cyberpunk design

### Features

Real-time monitoring with auto-refresh every 30 seconds

Detection cards showing:

Vehicle image / Cropped plate image

Arabic plate number

Confidence percentage

Timestamp

Today's vehicle count display

Caption: Dashboard showing detection results

### Responsive Design

The dashboard uses a responsive grid layout that adapts to different screen sizes, from mobile to desktop.

## Hardware Setup

### Components List

| Component | Specification | Quantity |
| --- | --- | --- |
| Microcontroller | ESP32-CAM (AI-Thinker) | 1 |
| Camera Module | OV2640 | 1 |
| Distance Sensor | HC-SR04 Ultrasonic | 1 |
| Voltage Regulator | 7805 (5V output) | 1 |
| Batteries | 18650 Li-ion (3.7V) | 2 |
| Resistors | 1kΩ (1/4W) | 3 |
| FTDI Programmer | USB-to-Serial | 1 |

### GPIO Pin Assignments

| GPIO | Function | Connection |
| --- | --- | --- |
| GPIO 13 | TRIG Output | HC-SR04 trigger pin |
| GPIO 12 | ECHO Input | HC-SR04 echo (via voltage divider) |
| GPIO 4 | Flash LED | Built-in LED (PWM controlled) |
| GPIO 0 | Boot Mode | GND for programming |

### Power Supply

Two 18650 batteries connected in series provide approximately 7.4V. The 7805 voltage regulator steps this down to stable 5V for the ESP32-CAM and HC-SR04.

| 7805 Pin | Connection |
| --- | --- |
| Input | Battery positive (~7.4V) |
| Ground | Common ground |
| Output | 5V to ESP32-CAM and HC-SR04 |

### Voltage Divider Circuit

Critical: The HC-SR04 outputs 5V on ECHO, but ESP32 GPIO is 3.3V tolerant. A voltage divider is required.

ECHO (5V) ---[1kΩ]---+---[2kΩ]--- GND
                     |
                  GPIO 12 (3.3V)

Calculation:

V_out = V_in × (R2 / (R1 + R2))
V_out = 5V × (2kΩ / 3kΩ)
V_out = 3.33V ✓ (safe for ESP32)

### Detection Parameters

| Parameter | Value | Purpose |
| --- | --- | --- |
| Trigger Distance | 15 cm | Maximum distance to trigger capture |
| Minimum Distance | 2 cm | Ignore noise (very close readings) |
| Debounce Delay | 3000 ms | Wait between captures |
| Camera Resolution | SVGA (800×600) | With PSRAM |

### Distance Calculation

The ultrasonic sensor measures time-of-flight:

Distance = (Duration × Speed of Sound) / 2
Distance = (Duration × 0.034 cm/µs) / 2

Three readings are taken and averaged for stability.

### Camera Configuration

| Condition | Resolution | Quality | Frame Buffers |
| --- | --- | --- | --- |
| With PSRAM | SVGA (800×600) | 10 | 2 |
| Without PSRAM | VGA (640×480) | 12 | 1 |

### Flash LED Control

| Brightness | PWM Value |
| --- | --- |
| Off | 0 |
| 25% | 64 |
| 50% | 128 |
| 75% | 192 |
| 100% | 255 |

## Deployment Architecture

### Cloud Services

| Service | Platform | Purpose |
| --- | --- | --- |
| API Backend | Railway | Image processing, ML inference |
| Dashboard | Railway | Web interface |
| Database | Neon | PostgreSQL storage |
| Image Storage | Cloudinary | CDN hosting |

### Railway Platform

Railway provides container-based hosting with automatic deployments from Git.

API Container Configuration:

FROM python:3.11-slim
RUN apt-get update && apt-get install -y libgl1 libglib2.0-0
COPY requirements.txt .
RUN pip install --no-cache-dir -r requirements.txt
COPY api/ ./api/
COPY *.pt .
CMD uvicorn main:app --host 0.0.0.0 --port $PORT

Features:

Auto-scaling for variable traffic

Automatic HTTPS/SSL certificates

Environment variable management

Built-in logging and monitoring

### Neon PostgreSQL

Serverless PostgreSQL with:

Auto-suspend when inactive

Auto-scaling compute

Connection pooling

SSL/TLS encryption

### Database Schema

| Field | Type | Description |
| --- | --- | --- |
| id | Integer | Primary key |
| car_image | String | Vehicle image URL |
| lp_image | String | Plate crop URL |
| lp_number | String | Arabic plate text |
| confidence | Float | Detection confidence |
| created_at | DateTime | Timestamp (Egypt TZ) |

### Cloudinary CDN

| Folder | Content |
| --- | --- |
| lpr/cars | Full vehicle images |
| lpr/plates | Annotated plate crops |

Benefits:

Global CDN distribution

Automatic image optimization

Secure HTTPS URLs

### Environment Variables

| Variable | Service | Description |
| --- | --- | --- |
| DATABASE_URL | Neon | PostgreSQL connection string |
| CLOUDINARY_CLOUD_NAME | Cloudinary | Account identifier |
| CLOUDINARY_API_KEY | Cloudinary | API authentication |
| CLOUDINARY_API_SECRET | Cloudinary | API secret key |
| PORT | Railway | Server port (auto-set) |
| VITE_API_URL | Dashboard | Backend API URL |

## System Data Flow

### Complete Data Flow Diagram

┌─────────────┐     HTTPS      ┌─────────────┐
│  ESP32-CAM  │───────────────▶│ Railway API │
└─────────────┘                └──────┬──────┘
                                      │
                   ┌──────────────────┼──────────────────┐
                   │                  │                  │
                   ▼                  ▼                  ▼
            ┌───────────┐      ┌───────────┐      ┌───────────┐
            │Cloudinary │      │   Neon    │      │ Dashboard │
            │ (Images)  │      │(Database) │      │  (React)  │
            └───────────┘      └───────────┘      └───────────┘

### End-to-End Process

Vehicle Detection: Ultrasonic sensor detects object within 15cm

Image Capture: ESP32-CAM captures JPEG image with flash

API Request: Image sent via HTTPS POST to Railway backend

ML Processing:

YOLOv10 detects license plate region

Plate cropped and resized to 640×640

Arabic OCR extracts characters

Characters sorted right-to-left

Storage: Images uploaded to Cloudinary, record saved to PostgreSQL

Dashboard: React app fetches and displays results every 30 seconds

### Communication Protocols

| Connection | Protocol | Security |
| --- | --- | --- |
| ESP32 → API | HTTPS | TLS 1.3 |
| API → Database | PostgreSQL | SSL |
| API → Cloudinary | HTTPS | API Key |
| Dashboard → API | HTTPS | CORS |

### Performance Metrics

| Metric | Value |
| --- | --- |
| Detection Accuracy | 97.5% mAP@50 |
| OCR Precision | 95.88% |
| End-to-end Latency | ~2-3 seconds |
| Dashboard Refresh | 30 seconds |

## System Architecture Diagram

┌─────────────────────────────────────────────────────────────────────────────┐
│                     EGYPTIAN LICENSE PLATE RECOGNITION                       │
├─────────────────────────────────────────────────────────────────────────────┤
│                                                                             │
│   ┌───────────────┐                                    ┌───────────────┐    │
│   │               │           WiFi/Internet            │               │    │
│   │   ESP32-CAM   │═══════════════════════════════════▶│  Railway API  │    │
│   │ + Ultrasonic  │           POST /recognize          │   (FastAPI)   │    │
│   │               │                                    │               │    │
│   └───────────────┘                                    └───────┬───────┘    │
│          │                                                     │            │
│          │ Camera                                              │            │
│          │ Stream                              ┌───────────────┼───────┐    │
│          ▼                                     │               │       │    │
│   ┌───────────────┐                    ┌───────▼─────┐ ┌───────▼─────┐│    │
│   │  Local Debug  │                    │ Cloudinary  │ │    Neon     ││    │
│   │  http://IP/   │                    │   (CDN)     │ │ PostgreSQL  ││    │
│   └───────────────┘                    └─────────────┘ └─────────────┘│    │
│                                                │               │       │    │
│                                                └───────────────┼───────┘    │
│                                                                │            │
│                                                        ┌───────▼───────┐    │
│                                                        │   Dashboard   │    │
│                                                        │   (React)     │    │
│                                                        │               │    │
│                                                        │ GET /dashboard│    │
│                                                        └───────────────┘    │
│                                                                             │
└─────────────────────────────────────────────────────────────────────────────┘

## Demonstration

The project is demonstrated using a miniature maket with a toy car displaying an Egyptian license plate. When the car approaches the ultrasonic sensor, the system automatically captures and processes the plate.

Demonstration Components:

Toy car with printed Egyptian license plate

ESP32-CAM mounted at plate level

Ultrasonic sensor positioned to detect approaching vehicles

Battery-powered portable operation

This allows showcasing the complete detection flow without requiring actual vehicles.

Caption: Maket demonstration showing hardware setup with toy car

## Scaling Considerations

### Current Resource Limits

| Resource | Limit |
| --- | --- |
| API Memory | 512MB |
| Request Timeout | 60 seconds |
| Database Connections | 100 pooled |
| Image Size | 10MB max |

### Performance Optimizations

Model Lazy Loading: ML models loaded on first request

Connection Pooling: Reuse database connections

Image Compression: JPEG quality optimization

CDN Caching: Static assets cached globally

### Reliability Features

WiFi auto-reconnection on ESP32

Database connection pooling

Image fallback handling

Error state recovery

## Cost Structure

| Service | Pricing Model |
| --- | --- |
| Railway | Usage-based (memory × time) |
| Neon | Free tier available, then usage-based |
| Cloudinary | Free tier (25GB), then storage + bandwidth |